Guide for Filling Out This Table

1. Function Tag: The exact name of the configuration node in the XML (do not change this).

2. Description & Allowed Values: This is a Reference column. It explains what the function does and what inputs are valid.

3. Desired Settings: (Action Required): Write specific values here.

- Example: `count: 10` or `mode: SWEEP`.

4. Applicable Stage: (Action Required) Specify which test stage this applies to:

- RF1/ RF2/NFR

5. Enable: (Action Required)

- TRUE: Activate function.

- FALSE: Deactivate function.

1.Common Attributes

Every configuration block contains <Package> elements. These are standard attributes you will see:

| Attribute | Value | Description |
| --- | --- | --- |
| name | String | “SUSER”: Local / Engineering Mode “Product P/N”: ENG_xxxx-AP1-RFx |
| enable | TRUE/FALSE | TRUE: Activates this feature for the specific package |

DC Contact Open/Short (O/S) Test Configuration

Function Tag 1: <DC_CONTACT_MODE_ENABLE> (The Switch)
Function Tag 2: <DC_CONTACT_MODE_SETTING_OVERWRITE> (The Settings)
Objective: Customize the electrical parameters for Open/Short testing on MIPI and DC pins prior to RF testing.
Applicable Test Stage: RF1 / RF2 / NFR

1. Enable/Disable (The Switch)

Before configuring parameters, ensure the feature is turned on for your package.

Tag: <DC_CONTACT_MODE_ENABLE>

| Package | Enable | Note |
| --- | --- | --- |
| SUSER | TRUE | Enable for local engineering debug |
| Production (e.g., 8288-AP1) | FALSE | Typically disabled in production unless debugging |

2. Test Methodology

(Applies if Enabled above)

- MIPI Pins (VIO, SCLK, SDATA):

  - Method: Force Current, Measure Voltage (FORCEI).

  - Goal: Detect Open and Short circuits.

- DC Pins (Vcc, Vbatt, Vdd):

  - Method: Force Voltage, Measure Current (FORCEV).

  - Goal: Detect Short circuits only.

⚠️ Note: If a SHORT event is detected, the code will immediately turn OFF all DC supplies to prevent damage. An OPEN event will allow testing to proceed for data collection.

3. Configuration Parameters (Blue Box Settings)

Tag: <DC_CONTACT_MODE_SETTING_OVERWRITE>

To match the standard "Vettel NFR" methodology, use the following settings:

A. MIPI Pin Settings (Open/Short)

Apply these settings to pins like Vio1, SCLK, SDATA.

| Parameter (XML) | Value | Description |
| --- | --- | --- |
| TestMode | FORCEI | Force Current mode |
| ISource | 1.5e-6 | Force 1.5 µA (1.5e-6 Amps) |
| VSetHi (HighLimit) | 1.2 | Max expected voltage (Open circuit) |
| VSetLo (LowLimit) | -0.3 | Min expected voltage |
| ILevel | 2e-6 | Current compliance level |

B. DC Pin Settings (Short Only)

Apply these settings to pins like Vdd, Vbatt, Vcc.

| Parameter (XML) | Value | Description |
| --- | --- | --- |
| TestMode | FORCEV | Force Voltage mode |
| VSet | 0.1 | Force 100mV (0.1 V) |
| ISet | 10e-6 | Max current limit (10 µA) |
| HighLimit | 9.5e-6 | Fail threshold (Short if current > 9.5µA) |

4. XML Example (Paired)

<!-- STEP 1: Turn it ON -->

<DC_CONTACT_MODE_ENABLE>

<Package name="SUSER" enable="TRUE" />

<Package name="ENGR-8288-AP1-NF" enable="FALSE" />

</DC_CONTACT_MODE_ENABLE>

<!-- STEP 2: Configure it -->

<DC_CONTACT_MODE_SETTING_OVERWRITE>

<Pin Name="Vdd" VSet="0.1" ISet="10e-6" HighLimit="9.5e-6" TestMode="FORCEV"/>

<Pin Name="Vio1" VSetHi="1.2" VSetLo="-0.3" ISource="1.5e-6" ILevel="2e-6" TestMode="FORCEI"/>

<Delayms Value="80"/>

</DC_CONTACT_MODE_SETTING_OVERWRITE>

5. Expected Results (Reference)

| Parameter | Expected Value (Normal) | Fault Condition |
| --- | --- | --- |
| M_MeasV-VIO1 | ~0.3V to 0.55V | Open: ~1.2V / Short: ~0V |
| M_MeasV-SCLK/SDATA | ~0.3V to 0.7V | Open: ~1.2V / Short: ~0V |
| M_MeasI-Vbatt/Vdd | Near 0 (Negative OK) | Short: > 10µA |